Setting Up the Stimulus Screen

What to do when the respondent's monitor, its Windows display settings, or the lab computer changes

Martin's Family Clothing Consumer Behavior Research Laboratory, Jacksonville State University

Prepared October 13, 2026 for Dr. Mark J. Sciuchetti Jr. and lab technicians. Every menu path is quoted from the iMotions help center (read signed in on October 12, 2026) or, where marked, from what the lab computer "Lab - Alien" showed on October 12, 2026. Steps the lab adds on its own are labeled Lab convention.

# 1. When to use this procedure

Run the parts marked in the table below whenever one of these happens:

- A different monitor will show the stimuli, for example moving the respondent from "MARKS" to the Dell, or adding a new monitor.

- The Windows Display resolution or Scale changes on either monitor.

- The Smart Eye Aurora is moved to another monitor.

- A study is imported from another computer, or a study will be recorded on the second lab computer.

- Start Recording stops with "Inconsistent screen resolutions".

Current lab setup (October 12, 2026): respondents look at "MARKS" (2560 x 1440, 34.4 x 19.4 cm), which carries the Aurora. The Dell U2717D is the operator (experimenter) screen.

| What changed | A. Windows | B. Aurora setup | C. Global Settings | D. Each study | E. Test |
| --- | --- | --- | --- | --- | --- |
| New stimulus monitor | Yes | Yes | Yes | Yes | Yes |
| Scale or resolution changed | Yes | Yes | Yes | Yes | Yes |
| Aurora moved to another monitor | Check | Yes | If the stimulus monitor changed | If the stimulus monitor changed | Yes |
| Study imported from another computer |  |  |  | Yes | Yes |
| Only the operator monitor changed | Yes (same Scale) |  | Check |  | Yes |

# 2. Why the order matters

iMotions asks for the resolution and physical size of the stimulus screen because this “is important so that iMotions can correctly display the stimuli and calculate eye-tracking statistics for your setup” (Using the iMotions Study Builder). The Aurora also needs its own setup for each monitor and display setting. Each later step reads the one before it, so work in order: Windows first, then the Aurora setup, then Global Settings, then each study. If Windows settings change afterward, start again at Part A.

# Part A. Windows display settings

- Connect the second monitor to the computer that runs iMotions.

- Right-click the desktop and select Display settings.

- Select Extend these displays in the list next to Identify. Click Identify to see which number belongs to which monitor.

- Under Scale & layout, select each monitor in turn and set Scale to the same value on both. Lab convention: use 100% on both. The two-monitor article accepts any value if both match, and the survey article asks for 100% on all screens, so 100% satisfies both.

- Select the respondent's monitor and write down its Display resolution.

- Select the operator's monitor. Under Multiple displays, select Make this my main display.

You should see: two numbers when you click Identify; the operator's monitor marked as the main display; the same Scale value on both monitors.

Common mistake: leaving the two monitors at different Scale values. The article warns: “If the two monitors use different scale values, iMotions shows a resolution error when you start a recording.”

Watch the resolution after changing Scale. iMotions reads a scaled resolution: “A screen with 1920 × 1080 px resolution scaling at 125% will be discovered as having a 1536 × 864 px resolution.” So any change to Scale changes the resolution every study must use (Part D).

Source: How to use two monitors with iMotions

# Part B. Smart Eye Aurora setup

The Aurora article says a new setup is needed in the Smart Eye software if “Display settings are changed (e.g. different Windows display setting or monitors)”. Skip this part only for studies without the Aurora.

- Open the Smart Eye Setup Tool from the Windows Start menu and select Aurora.

- Click +New setup and select the monitor where the stimuli will be shown.

- Select the bracket type (the standard on-screen Metallic Bracket, or the under-screen Monitor Mount, an option for screens under 24.5 inches).

- Click the button that shows the center mark (the article prints it as Show center maker) and follow the placement instructions.

- Clean the bezel, then attach the mount flat and stable at the bottom center. It sits just below the display without covering it; the bezel must be at least 14 mm.

- Seat the Aurora on the mount (square pegs into square slots), glass facing the respondent.

- Press any key, click Next, align the guidelines to the edges of the eye tracker, and press Enter.

- Enter a setup name and click Finish. Lab convention: name it after the monitor, resolution, and Scale, for example MARKS_2560x1440_100pct. Then close the Smart Eye Setup Tool.

- In iMotions, open Preferences > Global Settings (Alt+G) > Eye tracking. Select Smart Eye Aurora, keep the Host Address at 127.0.0.1 (USB), click Test Connection, then OK.

You should see: the Eye Tracker Widget in the main window showing the connection and eye detection, with no “!” warning.

Common mistake: keeping the old setup after a display change. A “!” in the widget means no setup matches; the fix is creating a setup as above. If the Aurora does not track a new respondent, click Re-Profile.

Source: Setting up Aurora or AI-X in iMotions

# Part C. iMotions Global Settings: the stimulus screen

- Open Preferences > Global Settings and go to the Slide Show tab.

- Under Screen for stimulus display, select Use secondary screen. The Windows main display is the iMotions primary screen, so this shows the stimuli on the respondent's monitor.

- Check that the Resolution shown on the tab matches the respondent's monitor, then click OK.

You should see: the Resolution on the Slide Show tab equal to the resolution iMotions reports for the respondent's monitor.

Common mistake: selecting the primary screen. The stimuli then appear on the operator's monitor.

Source: iMotions Global Settings

# Part D. Each study that will be recorded

Screen-based studies. These steps are what Lab - Alien showed on October 12, 2026 (Project doc 14):

- Click the study in the Library and open STUDY SETTINGS > Screen.

- Click Edit size > Load from preset, choose the respondent's monitor under DETECTED SCREENS, and click Ok.

- If iMotions warns “The study contains survey slides. When changing the study resolution it is recommended to preview the survey slides.”, click OK.

- Click Update, then Preview study and look at every survey slide.

For full control, the Study Builder article recommends measuring the screen's width and height with a tape (in centimeters), entering the resolution in pixels, and saving it as a named custom screen, because automatic detection “is not always possible”. Lab convention: tape a card with the measured width, height, and resolution to each monitor.

You should see: Resolution, Width, and Height in STUDY SETTINGS matching the respondent's monitor, and survey slides that fit the screen.

Common mistake: changing a study that already holds recorded respondents. The screen values feed every gaze angle and fixation, so changing them under recorded data can make the eye tracking line up wrongly. Lab convention: clone the study (a clone keeps the design but not the respondents) and fix the clone.

Other study types:

- Remote Data Collection: not affected. iMotions fills in 1920 x 1080 and a 47.4 x 29.6 cm screen at 65 cm, “not accessible to the user for edits.”

- Glasses live recording and glasses import: not affected. They use the glasses' world camera.

- Sensor Sync for Glasses (for example Snack Launch Lab - Shelf Detective): affected. It uses the Global Settings screen, and the article recommends updating the study settings when the study runs on a computer with a different resolution, so the synchronization stimulus displays correctly.

- Two computers: the distribution article requires the same respondent screen resolution on every computer, matching the study settings. Set the screen before saving the study for distribution, and do not change it afterward.

Source: Using the iMotions Study Builder (Configuration of the screen size)

# Part E. Test, then record what you set

- On a copy of the study, add a test respondent and click Start Recording.

- Calibrate. Accept only “Excellent” or “Good”; recalibrate once or twice on anything less.

- Let one image and one survey slide play, then end the recording (Shift+F1).

- Write in the study README: the screen name, resolution, width and height, viewing distance, Windows Scale value, Smart Eye setup name, and iMotions version.

You should see: the recording starts without “Inconsistent screen resolutions”, and the stimuli fill the respondent's monitor.

# Items to verify on the lab computer

- Why iMotions reads the Dell as 3200 x 1800, when the U2717D's native resolution is 2560 x 1440. A likely cause is the two monitors using different Scale values, which Part A forbids.

- The Scale value on each monitor now, and what resolution iMotions reports for MARKS once both are equal. If it changes, every study needs Part D with the new value.

- That the Dell is the Windows main display and the Slide Show tab reads “Use secondary screen”.

- The exact labels on the Lab - Alien Slide Show tab (the article calls the setting “Screen for stimulus display”).

- The Smart Eye software version installed. The current Aurora article links installer 16.0.0.

- The viewing distance in the studies: the imported studies read 65 cm, and the calibration article positions the head about 60 cm from the screen.

# Sources

- How to use two monitors with iMotions. https://help.imotions.com/lab/sensor-setup/lab-setup-how-to-use-two-monitors-with-imotions

- iMotions Global Settings. https://help.imotions.com/lab/getting-started/imotions-global-settings

- Setting up Aurora or AI-X in iMotions. https://help.imotions.com/lab/sensor-setup/screen-based-eye-trackers/smart-eye/setting-up-aurora-or-ai-x-in-imotions

- Using the iMotions Study Builder. https://help.imotions.com/lab/getting-started/using-the-imotions-study-builder

- iMotions Survey Stimuli and Elements. https://help.imotions.com/lab/study-setup-and-design/stimuli/survey/imotions-survey-stimuli-and-elements

- Windows Settings Recommendations. https://help.imotions.com/lab/support-and-troubleshooting/windows-settings-recommendations

- Planning for Eye Tracker Calibration. https://help.imotions.com/lab/sensor-setup/screen-based-eye-trackers/planning-for-eye-tracker-calibration

- Distributing an iMotions Study for Data Collection on Multiple Computers. https://help.imotions.com/lab/study-setup-and-design/imotions-setup/distributing-an-imotions-study-for-data-collection-on-multiple-computers

- Clone, Save, and Import iMotions Studies. https://help.imotions.com/lab/study-setup-and-design/imotions-setup/clone-save-and-import-imotions-studies

Lab records: Project doc 14 (screen size mismatch found October 12, 2026) and doc 02a (Aurora pipeline).